# ME 315

THEORY OF MACHINES - DESIGN OF ELEMENTS

Fall 2023

Design Project

Group Submission

Group members

Chirag Bachani		Shaft system 1

Winston Zhao		Shaft system 2

Christopher Luey		Shaft system 3

Grade

On-time step dues			/5

Structural design, drawings	/40

Design analysis			/35

Report writing			/20

Total __________

# Final Project

Christopher Luey, Winston Zhao, Chirag Bachani

Introduction	4

Gear Train	4

Gears	5

Shafts	6

Bearings	7

Assembly Drawings	8

Shaft 1	8

Shaft 2	9

Shaft 3	10

Shaft Drawings	11

Shaft 1	11

Shaft 2	12

Shaft 3	13

Gear Drawings	14

Gear 1	14

Gear 2	15

Gear 3	16

Gear 4	17

Shaft One Analysis	18

Gearing	18

Force Analysis	20

Maximum Shear, Bending	21

Shaft Stress Analysis	21

Minimum Shaft Diameter	21

Infinite Life at Critical Cross Section	22

Yielding	23

Gear Stress Analysis	23

Bending	23

Contact	24

Factors of Safety for Bending and Contact	25

Bearing Selection	26

Expected Lifetime of Bearing	26

Keyways and Tolerances	27

Shaft Two Analysis	30

Gearing	30

Free Body Diagram	32

Force Analysis	34

Shear and Bending	34

Shaft Fatigue Analysis	34

Gear Stress Analysis	37

Material	37

Allowable Contact Stress	37

Allowable Bending Stress	39

Design Bending Stress	39

Bearing Selection	41

Expected Lifetime of Bearing	41

Keyways and Tolerances	43

Shaft Three Analysis	44

Gearing	44

Geometry	45

Force Analysis	46

Maximum Shear, Bending	47

Shaft Stress Analysis	48

Minimum Shaft Diameter	48

Infinite Life at Critical Cross Section	48

Yielding	49

Gear Stress Analysis	50

Bending	50

Contact	51

Factors of Safety for Bending and Contact	52

Bearing Selection	53

Expected Lifetime of Bearing	53

Keyways and Tolerances	54

# Introduction

In our team's engineering project, we designed a robust two-stage gear transmission system depicted in the accompanying figure, aiming to efficiently transmit power from input shaft A to output shaft C with a speed reduction. The system, comprising input shaft A driven by two electric motors, center shaft B with corresponding gears, and output shaft C with a coupling set, was carefully arranged, considering the spatial relationship between A and C. Operating at 2600 rpm, the input motors delivered 70KW to shaft A, with the goal of achieving an output speed (n3) of around 400 ±3 rpm, assuming 100% overall efficiency. Our design incorporated fatigue stress concentration factors to address bending and torsional stresses, and determined geometry using a factor of safety of n=1.1. With a commitment to durability, the transmission system is expected to function continuously for at least 5 years, 7 days a week, 52 weeks a year, and 8 hours a day, with a reliability rate of 99%.

# Gear Train

| Part | Gear 1 | Gear 2 | Gear 3 | Gear 4 |
| --- | --- | --- | --- | --- |
| N [teeth] | 19 | 47 | 27 | 71 |
| RPM [rev/min] | 2600.000 | 1051.064 | 1051.064 | 399.700 |
| Power [kW] | 70.0 | 70.0 | 70.0 | 70.0 |

| Gear Train Result |  |
| --- | --- |
| Ratio | 6.5049 |
| Output Speed (RPM) | 399.7003 |
| % Error | -0.0749% |

# Gears

| Gear Properties |  |
| --- | --- |
| Material | AISI 1045 Steel |
| Module [mm] | 5.00 |
| Pitch Angle [Deg] | 20.00 |
| Addendum [mm] | 5.00 |
| Dedendum [mm] | 6.25 |
| Clearance [mm] | 1.25 |
| Tooth Depth [mm] | 11.25 |

|  | Gear 1 | Gear 2 | Gear 3 | Gear 4 |
| --- | --- | --- | --- | --- |
| Material Grade | 2 | 1 | 2 | 1 |
| Hardness [HB] | 500 | 400 | 500 | 400 |
| Pitch Diameter [mm] | 95.0 | 235.0 | 135.0 | 355.0 |
| Outer Circle [mm] | 105.0 | 245.0 | 145.0 | 365.0 |
| Root Circle [mm] | 83.0 | 223.0 | 123.0 | 343.0 |
| Face Width [mm] | 65.0 | 60.0 | 65.0 | 60.0 |
| Centerline Distance [mm] | 165.0 |  | 245.0 |  |
| Bending Factor of Safety | 4.1069 | 3.0405 | 2.7465 | 2.0373 |
| Contact Stress Factor of Safety | 1.4576 | 1.1201 | 1.4859 | 1.1462 |

# Shafts

|  | Shaft 1 | Shaft 2 | Shaft 3 |
| --- | --- | --- | --- |
| Material | AISI 1045 Carbon Steel |  |  |
| RPM [rev/min] | 2600.0000 | 1051.0638 | 399.7003 |
| Max Bending Magnitude [Nm] | 205.7115 | 451.2470 | 343.7668 |
| Max Torque [Nm] | 257.096 | 635.975 | 1672.380 |
| Minimum Shaft Diameter [mm] | 17.000 | No pure torque | 40.000 |
| Critical Cross Section Location | Gear 1 Center | Gear 3 Center | Gear 4 Center |
| Shaft Diameter at Critical Cross Section [mm] | 45.000 | 60.000 | 55.000 |
| Goodman Factor of Safety | 1.8467 | 1.8236 | 1.6866 |
| Yielding Factor of Safety | 7.3976 | 7.3851 | 6.3999 |
| Lifetime | Infinite (> 106 Cycles) |  |  |

# Bearings

|  | Shaft 1 |  | Shaft 2 |  | Shaft 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| As shown in Assembly Drawing | Left | Right | Left | Right | Left | Right |
| Radial Load [N] | 1645.692 | 4114.230 | 8204.490 | 6835.208 | 6875.336 | 3151.196 |
| Thrust Load [N] | 0 |  | 0 |  | 0 |  |
| Equivalent Load Used in Calculation [N] | 4114.230 |  | 8204.490 |  | 6875.336 |  |
| C [N] | 54081.734 |  | 79744.149 |  | 48414.597 |  |
| Bearing (SKF) | 6309 |  | 6312 |  | 6311 |  |
| Bore [mm] | 45 |  | 65 |  | 55 |  |
| OD [mm] | 100 |  | 140 |  | 120 |  |
| Width [mm] | 25 |  | 33 |  | 29 |  |
| C, bearing [N] | 55300 |  | 97500 |  | 74100 |  |
| C0, bearing [N] | 31500 |  | 60000 |  | 45000 |  |
| Expected Life [Mil Cycles] | 2428.3404 |  | 1678.2594 |  | 1251.9091 |  |
| Expected Life [Yrs] | 5.3455 |  | 9.1288 |  | 17.9265 |  |

# Assembly Drawings

## Shaft 1

## Shaft 2

## Shaft 3

# Shaft Drawings

## Shaft 1

## Shaft 2

## Shaft 3

# Gear Drawings

## Gear 1

## Gear 2

## Gear 3

## Gear 4

# Shaft One Analysis

## Gearing

Given: n1 = 2600 rpm, H = 70000 W, n3 (ideal) = 400, n1/n3 (ideal) = 6.5

Choose gear teeth (odd number, indivisible):

N1 = 19

N2 = 47

N3 = 27

N4 = 71

Choose standard module and pitch angle for off the shelf ordering:

m = a = 0.005 [m]

b = 1.25m = 0.00625 [m]

ϕ = 20 degree

Calculate transmission ratio:

n1/n3 = N4*N2/(N1*N3) = 6.5049

n3 = 2600/(n1/n3) = 399.7003 rpm

% error = |(n3-400)/400| = -0.0749%

Gear 1:

Geometry

N1 = 19

Pitch Diameter: dp=N1m=0.0950 [m]

Pitch Radius: rp=dp/2=0.0480 [m]

Face Width: bw=12m=0.0650 [m]

Forces

Note: Forces acting from Gear 1 to Gear 2 are the same as from Gear 2 to Gear 1

Torque: T=60H2πn1=257.096 Nm

Gear Forces:

WT12=T/rp=5412.557 N

W12=WT12/cos(ϕ)=5759.923 N

Wr12=W12sin(ϕ)=1970.010 N

Note: Torque occurs in the same direction on both sides of the shaft, coming from the coupling.

## Force Analysis

From the gearing section the following are solved:

WT12=5412.557 N

Wr12=1970.010 N

From geometry we know the following:

a=55 mm; b+c =120 mm

d/2=177.5 mm

Derived from static equilibrium, solve this system of equations:

0=Ay+By-Wt12

0=Az+Bz-Wr12

0=Wt12(a)-By(a+b+c)

0=Wr12(a)-Bz(a+b+c)

System of equations yields the following reactions with force directions as drawn:

Ay = 1546.4448 N

Az = 562.8599 N

By = 3866.1120 N

Bz = 1407.1497 N

A=Ay2+Az2=1645.6922 N

B=By2+Bz2=4114.2304 N

## Maximum Shear, Bending

Maximum bending occurs at cross section C:

My=Aya=70.3575 Nm

Mz=Aza=193.3056 Nm

M=My2+Mz2=205.7115 Nm

The shear magnitude at cross section C is:

Vy = Ay = 1546.4448 N

Vz = Az = 562.8599 N

V=Vy2+Vz2=1645.6922 N

## Shaft Stress Analysis

We first choose material:

| Material | E (Pa) | UTS (MPa) | Poisson Ratio | Ys (MPa) |
| --- | --- | --- | --- | --- |
| AISI 1045 Steel | 200000.0000 | 394.7200 | 0.2900 | 294.7400 |

### Minimum Shaft Diameter

Our factor of safety is n=1.1

Calculate the torque magnitude at the coupling:

T=Wt12d/2=257.096 Nm

Calculate the minimum shaft diameter:

τall=0.557(Ys)/n=156.9727 MPa

dmin=316T/2πτall=16.0967 mm

Consider keyway effect, round 5% increase:

dmin'=Round[dmin(1.05)]=17 mm

### Infinite Life at Critical Cross Section

Choose a shaft diameter of d = 45 mm at cross section C.

Solve for max bending torsion, and shear stresses at cross-section C experienced by point on the surface:

σbs=32Mπd3=22.9944 MPa

τT=16Tπd3=14.3691 MPa

τV=16V3πd2=3.4492 MPa

Given these stresses, calculate the mean and amplitude stresses for cyclic loading:

σa=σbs=22.9944 MPa

σm=0 MPa

τm=τT2=7.1845 MPa [The shaft turns on and off]

τa=τT2+τV=10.6337 MPa

This is a conservative estimate, since we use the shear and bending stress maximum amplitudes in calculating stress states, although these maximums are out of phase. Furthermore, we do not ignore shear (which is 30% of the mean shear).

Calculate Von-Mises given Kf = 3.000 and Kfs = 2.800

σvm-a=(Kfσa)2+3(Kfsτa)2=86.1289 MPa

σvm-m=(σm)2+3(τa)2=12.4440 MPa

Calculate Infinite Criteria given chosen material:

kf=1.58*Sut-0.085=0.9220 [Grinding]

ks=1.189*d-0.112=0.7763 [Size Factor]

kr=0.82 [99% Reliability]

kt=km=1.000

Se'=0.5(UTS)=282.500 MPa

ns=[σvm-akfkskrktkmSe'+σvm-mSut]-1=1.8467 >1.1		[Chosen shaft diameter at C is safe]

### Yielding

ny=[σvm-aSy+σvm-mSy]-1=7.3976> 1.1     [Chosen shaft diameter does not pass yield line]

## Gear Stress Analysis

Choose material for gear:

| Gear Materia | hardness | E (MPa) | Poisson Ratio |
| --- | --- | --- | --- |
| AISI 1045 Carbon Steel | 400.0000 | 200000.0000 | 0.2850 |

### Bending

Calculate number of shaft cycles:

t=5 year*52 weekyear*7 dayweek * 8 hrday * 60 minhr=873600 min

Rotation speed n1= 2600 RPM.

N=t*n1=2271360000 Cycles

Calculate allowable bending stress:

Yn=1.683(N)-0.0323=0.8392

Sb=0.533(HB)+88.3=464.5000MPa

kt=kr=1.000

σb, all=SbYnktkr=389.8179 MPa

Calculate actual bending stress:

Yj=0.350	[From Class Plot]

ka=1.000

ks=1.000	[module = 5 mm]

km=1.100	[Gear]

ki=1.000	[Non Idler]

kb=1.000

Calculate Kv:

V=πnd/60=12.9329 [m/s]

Qv=6	for commercial gears

B=(12-Qv)2/34=0.8255

A=50+56(1-B)=59.7730

Kv=(A+200VA)B=1.6623

σ b, actual=WtmbwYjkakskmkvkikb=94.9170 MPa

Comparing the allowable to the actual bending stress:

nb=σall/σb=4.1069>1.1

The design is sufficient for bending stress since nb > n = 1.1

### Contact

Calculate allowable contact stress:

HBpinion=500 (HB)

HBgear=400 (HB)

ZN=2.466N-0.056=0.7380

HBpHBg=1.25

A' = (8.89*10-3)(HBpHBg)-8.29*10-3	since 1.2≤HBpHBg=1.25≤1.7

NgNp=2.6296

CH=1.0+A'(NgNp-1.0)=1.0042

KT=1

KR=1

Sc=2.22*HBg+200=1437 MPa

σc, all=Sc_ZNCHgKTKR=1060.4686 MPa

Calculate actual contact stress:

Ka=1	[No shock]

Ks=1

KE=21-vp2Ep+1-vg2Eg=466563.1271     [Pa]

Km, gear=1.1

Kv=1.6623 		[From bending calculation]

ϕ=20 deg; dp=0.095 m; dg=0.235 m

I=πcos(ϕ)sin(ϕ)1+dpdg=0.7190

σc,actual=KEWtbwdpIKaKsKmKv=727.5603 MPa

### Factors of Safety for Bending and Contact

σc,actual=727.5603 MPa

σc, all=1060.4686 MPa

σ b, actual=94.9170 MPa

σb, all=389.8179 MPa

nc=σc, allσc,actual=1.4576>1.1		[Safe for contact stress]

nb=σb, allσb,actual=4.1069>1.1		[Safe for bending stress]

## Bearing Selection

h=(5 year)(364 dayyear)(8hrday)=14560 hr	[life hours]

n3 = 2600 RPM	[rotation speed]

L10=60(h)(n3)(10-6)=2271.3600 Million Revs

AP=1.000			[No Shock]

m=3 [Ball Bearing]

Pr=Max(A,B)=4114.2304 N		[A and B calculated in Force Analysis section]

X=1.000

P=PrX=4114.2304 N 		[No Axial Force]

C=PL101/m=54081.7335 N

Given the calculated C, I specify the following bearing from SKF:

Reference: https://www.skf.com/us/products/rolling-bearings/ball-bearings/deep-groove-ball-bearings/productid-6309

This bearing is safe because the bearing specification C is less than our calculated C. Recalculation is not necessary since there is no thrust force and radial factor X is constant, not a function of e or C0.

### Expected Lifetime of Bearing

L=(CP)m=2428.3404 mil rev	[using C=55300 N from SKF]

Life=L(106)(n1)-1(60minhr)-1(8hrday)-1(7dayweek)-1(52weekyr)-1=5.3455 yr

Bearing does not require maintenance for intended operation of 5 years.

## Keyways and Tolerances

Coupling Keyway (40 mm) and Gear Keyway (50 mm):

| Shaft diameter d (mm) | Key width W x height H |
| --- | --- |
| 38 < d <= 44 | 12 x 8 |
| 44 < d <= 50 | 14 x 9 |

Fits:

Bearing and Shaft: Interference fit (H7/p6), bearing should be tightly fit with shaft for fluid rotation.

Gear and Shaft: Transition fit (H7/k6), shaft rotation will be transmitted from gear torque through interlocked key.

Source: https://amesweb.info/fits-tolerances/tolerance-calculator.aspx

# Shaft Two Analysis

## Gearing

Given: n1 = 2600 rpm, n3 = 400 rpm ideal, n1/n3 = 6.5 ideal

N1 = 19

N2 = 47

N3 = 27

N4 = 71

m = 0.005 m

Phi = 20 degree

Power H = 70000 W

n1/n3 = N4*N2/(N1*N3) = 6.5049

n3 = 2600/(n1/n3) = 399.7003 rpm

% error = (n3-400)/400 = -0.0749%

Gear 2:

Geometry

N2 = 47

Pitch Diameter: dp = N*m = 0.235 m

Pitch Radius: rp = dp/2 = 0.1175 m

Outer Diameter: do = dp + 2*m = 0.245 m

Root Diameter: dr = dp - 2*1.25*m = 0.223 m

Face Width: bw = 12*m = 0.06 m

Forces

Torque: T2 = 60*H/(2*pi*n2) = 635.975Nm

Wt23 = T2/rp = 5412.557 N

W23 = Wt23/cos(phi) = 1970.01 N

Wr23 = W23*sin(phi) = 5759.923 N

Gear 3:

Geometry

N3 = 27

Pitch Diameter: dp = N*m = 0.125 m

Pitch Radius: rp = dp/2 = 0.0675 m

Outer Diameter: do = dp + 2*m = 0.145 m

Root Diameter: dr = dp - 2*1.25*m = 0.123 m

Face Width: bw = 13*m = 0.065 m; Pinion has larger bw than gear

Forces

Torque: T3 = T2 = 60*H/(2*pi*n3) =  635.975 Nm

Wt34 = T3/rp = 9421.858 N

W34 = Wt34/cos(phi) = 10026.532 N

Wr34 = W34*sin(phi) = 3429.276 N

## Free Body Diagram

Rotation Speed:  n2 = n1*N1/N2 = 1051.0638

Set shaft diameter: d2 = 0.06 m

a = 0.0550 m 	[bw3/2 + bearing thickness/2 + spacing]

b = 0.0700 m	[bw3/2 +bw2/2 + spacing]

c = 0.0500 m	[bw2/2 + bearing thickness/2 + spacing]

## Force Analysis

Shaft 2 Reaction Forces

By=Wt,43*a+Wt,12*(a+b)a+b+c=6827.2674 N

Ay=Wt,43+Wt,12-By= 8007.1475 N

Bz=Wr,43*a+Wr,12*(a+b)a+b+c= 1788.6436 N

Az=Wr,43-Wr,12+Bz=329.3773 N

RA=Ay2+Ay2=8204.4900 N

RB=By2+By2=6835.2081 N

## Shear and Bending

Maximum bending occurs at C:

Mz = Ay * a = 440.3931 Nm

My = Az * a = 98.3754 Nm

M=My2+Mz2=451.2470Nm

The shear at point C is:

Vy = Ay = 8007.1475 N

Vz = Az = 1788.6436 N

V = Sqrt(Vy^2 + Vz^2) = 6875.3362 N

### Shaft Fatigue Analysis

Solve for bending torsion, and shear stresses at Point 1 at  cross-section C:

σbs=32Mπd3=21.2795 MPa

τT=16Tπd3=14.9954 MPa

τV=16V3πd2=3.8690 MPa

Given these stresses, calculate the mean and amplitude stresses for cyclic loading:

σa=σbs=21.2795 MPa

σm=0 MPa	[ignore axial normal stress due to low magnitude]

τm=τt2=7.4977 MPa	[on/off]

τa=τT2+τv=11.3667 MPa

Calculate Von-Mises given Kf = 3.000 and Kfs = 2.800.

Note: although max σbs and max τts won’t coexist at the same point mechanically, we take both magnitudes into account for a conservative design on calculating the factor of safety for our shafts.

σvm-a=(Kfσa)2+3(Kfsτa)2=91.2798 MPa

σvm-m=(σm)2+3(τm)2=16.8598 MPa

Calculate Infinite Criteria given chosen shaft material:

| Material | E | UTS | Poisson Ratio | Ys |
| --- | --- | --- | --- | --- |
| AISI 1045 Steel | 200000.0000 | 394.7200 | 0.2900 | 294.7400 |

kf=1.58*Sut-0.085=0.9220 [Grinding]

ks=1.189*d-0.112=0.7517 [Size Factor]

kr=0.82 	[99% Reliability]

kt=km=1.000	[ignore temperature and miscellaneous factors]

Se'=0.5(UTS)=282.500 MPa

Goodman

ns=[σvm-akfkskrktkmSe'+σvm-mSut]-1=1.8236 > 1.1 	[The shaft design is safe]

Yield

ny=σvm-maxSy=7.3851

## Gear Stress Analysis

### Material

| Gear Material |  | hardness | E (MPa) | Poisson Ratio | Type |
| --- | --- | --- | --- | --- | --- |
| Gear 3 | AISI 1045 Carbon Steel | 500.0000 | 200000.0000 | 0.2850 | pinion |
| Gear 2 | AISI 1045 Carbon Steel | 400.0000 | 200000.0000 | 0.2850 | gear |

### Allowable Contact Stress

HBgear=400 (HB)

HBpinion=500 (HB)

ZN=2.466N-0.056=0.7764

HBpHBg=1.25

A' = (8.89*10-3)(HBpHBg)-8,29*10-3	since 1.2≤HBpHBg=1.25≤1.7

CH=1.0+A'(NgNp-1.0)=1.0042

KT=1

KR=1

Gear 2:

Sc2=2.22*HBg+200=1088 MPa

σall,2=Sc_ZNCHgKTKR=848.2032 MPa

Gear 3:

Sc3=2.4*HBp+237=1437 MPa

σall,3=Sc_ZNKTKR=1115.6426 MPa

Design Contact Stress

Gear 2:

Ka=1	for no shock

Ks=1

KE=21-vp2Ep+1-vg2Eg=466563.1271     [Pa]

Km, gear=1.1

Calculate Kv:

Qv=6	for commercial gears

B=(12-Qv)2/34=0.8255

A=50+56(1-B)=59.7730

V2=π*n2*dp260=12.9329 m/s

Kv=(A+200VA)B=1.6623

Calculate Design Contact Stress

σc,actual=KE(WtbwdpIKaKsKmKv)=725.0297MPa

Gear 3:

Ka=1	for no shock

Ks=1

KE=21-vp2Ep+1-vg2Eg=466563.1271     [Pa]

Km, pinion=1.2

Calculate Kv:

Qv=6	for commercial gears

B=(12-Qv)2/34=0.8255

A=50+56(1-B)=59.7730

V3=π*n2*dp360=7.4295 m/s

Kv=(A+200VA)B=1.5081

Calculate Design Contact Stress

σc,actual=KE(WtbwdpIKaKsKmKv)=760.4055 MPa

### Allowable Bending Stress

Calculate number of shaft cycles:

hrs=5 year*52 weeks*7 days * 8 hrs * 60 minutes=873600 mins

N=hrs*shaft speed(rpm)=918209362 Cycles

Gear 2:

Calculate allowable bending stress

Yn=1.683(N)-0.0323=0.8915

Sb=0.533(HBgear)+88.3=301.5000 MPa

kt=kr=1.000

σall,b=SbYnktkr=260.5364 MPa

Gear 3:

Calculate allowable bending stress

Yn=1.683(N)-0.0323=0.8915

Sb=0.533(HBpinion)+88.3=464.5000 MPa

kt=kr=1.000

σall,b=SbYnktkr=401.3902 MPa

### Design Bending Stress

Gear 2:

Yj=0.420

ka=1.000

ks=1.000	[module == 5mm]

km=1.100	[gear]

ki=1.000	[not idler]

kb=1.000

Kv=(A+200VA)B=1.6623

σb=WtmbwYjkakskmkvkikb=78.5482 MPa

Gear 3:

Yj=0.350

ka=1.000

ks=1.000 	[module == 5mm]

km=1.200	[pinion]

ki=1.000	[not idler]

kb=1.000

Kv=(A+200VA)B=1.5081

σb=WtmbwYjkakskmkvkikb=149.8943 MPa

Factors of Safety

nc,2=σall,c2σc2=1.1699

nc,3=σall,c3σc3=1.4672

nb,2=σall,b2σb2=3.3169

nb,3=σall,b3σb3=2.6778

The design is good

## Bearing Selection

h=(5 year)(364 dayyear)(8hrday)=14560 hr	[life hours]

n2 = n1*N1/N2 = 1051.0638 rpm			[rotation speed]

L10=60(h)(n3)(10-6)=918.2094 million cycles

AP=1.000		[no shock]

m=3 		[Deep Groove Ball Bearing]

Pr=Max(A,B)=8204.4900 N		[A and B calculated in Force Analysis section]

X=1.000

Initial Selection

P=PrX=8204.4900 N 		[No Axial Force]

C=PL101/m=79744.1494 N

Choose SKF 6312 with C = 97500 N, C0 = 60000 N

No reselection necessary since no thrust load

e=0.22	because PaCo=0

PaPr=0 < e

- X=1

- Y=0

Pick SKF 6312 for Final Bearing Selection with following specifications

| Bearing Spec | C (N) | C0 (N) | d (m) | D (m) | W (m) |
| --- | --- | --- | --- | --- | --- |
| SKF 6312 | 97500.0000 | 60000.0000 | 0.0650 | 0.1400 | 0.0330 |

Reference: https://www.skf.com/group/products/rolling-bearings/ball-bearings/deep-groove-ball-bearings/productid-6312

### Expected Lifetime of Bearing

L=(CP)m=1678.2594 mil rev	[using C=97500 N from SKF]

Life=L(106)(n2)-1(60minhr)-1(8hrday)-1(7dayweek)-1(52weekyr)-1=9.1288 yr

Bearing does not require maintenance for intended operation of 5 years.

## Keyways and Tolerances

Gear Keyway (65 mm):

| Shaft diameter d (mm) | Key width W x height H |
| --- | --- |
| 58 < d <= 65 | 18 x 11 |

Fits:

Bearing inner ring and Shaft:

Interference fit (H7/p6) for hole basis. Interference needed for inner ring to rotate with shaft

Gear and Shaft:

Transition fit (H7/k6) for hole basis. Locational transition since shaft rotation will be transmitted from gear torque through interlocked key.

# Shaft Three Analysis

## Gearing

Given: n1 = 2600 rpm, H = 70000 W

Choose gear teeth (odd number, indivisible):

N1 = 19

N2 = 47

N3 = 27

N4 = 71

Choose standard module m=5mm and pitch angle phi = 20 deg for off the shelf ordering:

m = a = 0.005 [m]

b = 1.25m = 0.00625 [m]

ϕ = 20 degree

Calculate transmission ratio:

n1/n3=(N4)(N2)(N1)-1(N3)-1=6.5049

n3 = 2600/(n1/n3) = 399.7003 rpm

% error = |(n3-400)/400| = 0.0749%

Gear 4:

Geometry

N4 = 71 [Teeth]

Pitch Diameter: dp=N4m=0.3550 [m]

Pitch Radius: rp=dp/2=0.1775 [m]

Face Width: bw=12m=0.0600 [m]

Forces

Torque: T=60H2πn3=1672.380 Nm

Gear Forces: WT34=T/rp=9421.8581 N

W34=WT34/cos(ϕ)=10026.532

Wr34=W34sin(ϕ)=3429.276 N

## Geometry

## Force Analysis

From the gearing section the following are solved:

Wr34=3429.2759 N

Wt34=9421.8581 N

From geometry we know the following:

a=55 mm; b+c =120 mm

d/2=177.5 mm

Derived from static equilibrium, solve this system of equations:

0=Ay+By-Wt34

0=Az+Bz-Wr34

0=Wt34(a)-By(a+b+c)

0=Wr34(a)-Bz(a+b+c)

System of equations yields the following reactions with force directions as drawn:

Ay = 6460.7027 N

Az = 2351.5035 N

By = 2961.1554 N

Bz = 1077.7724 N

A=Ay2+Az2=6875.3362 N

B=By2+Bz2=3151.1958 N

## Maximum Shear, Bending

Maximum bending occurs at cross section C:

Mmax,y=Aya=117.5752 Nm

Mmax,z=Aza=323.0351 Nm

M=My2+Mz2=343.7668 Nm

The shear magnitude at cross section C is:

Vy = Ay = 6460.7027 N

Vz = Az = 2351.5035 N

V=Vy2+Vz2=6875.3362 N

## Shaft Stress Analysis

We first choose material:

| Material | E [Pa] | UTS [MPa] | Poisson Ratio | Ys [MPa] |
| --- | --- | --- | --- | --- |
| AISI 1045 Medium Carbon Steel | 200000.0000 | 565.0000 | 0.2850 | 310.0000 |

### Minimum Shaft Diameter

Our factor of safety is n=1.1

Calculate the torque magnitude at the coupling:

T=Wt34d/2=1672.380 Nm

Calculate the minimum shaft diameter:

τall=0.557(Ys)/n=156.9727 MPa

After unit conversion:

dmin=316Tπτall=37.858 mm

Consider keyway effect, round 5% increase [conservative]:

dmin'=Round[dmin(1.05)]≈40 mm

### Infinite Life at Critical Cross Section

Choose a shaft diameter of d = 55 mm at cross section C.

Solve for max bending torsion, and shear stresses at cross-section C experienced by point on the shaft surface:

σbs=32Mπd3=21.0463 MPa

τT=16Tπd3=19.4680 MPa

τV=16V3πd2=3.8585 MPa

Given these stresses, calculate the mean and amplitude stresses for cyclic loading:

σa=σbs=21.0463 MPa

σm=0 MPa

τm=τT2=9.7340MPa [The shaft turns on and off]

τa=τT2+τV=13.5925 MPa

This is a conservative estimate, since we use the shear and bending stress maximum amplitudes in calculating stress states, although these maximums are out of phase. Furthermore, we do not ignore shear (which is 30% of the mean shear).

Calculate Von-Mises given Kf = 3.000 and Kfs = 2.800 [conservative estimate]:

σvm-a=(Kfσa)2+3(Kfsτa)2=91.2798 MPa

σvm-m=(σm)2+3(τa)2=16.8598 MPa

Calculate Infinite Criteria given chosen material:

kf=1.58*UTS-0.085=0.9220 [Grinding]

ks=1.189*d-0.112=0.7590 [Size Factor]

kr=0.82 [99% Reliability]

kt=km=1.000

Se'=0.5(UTS)=282.500 MPa

ns=[σvm-akfkskrktkmSe'+σvm-mUTS]-1=1.6866 >1.1		[Chosen shaft diameter at C is safe]

### Yielding

ny=[σvm-aSy+σvm-mSy]-1=6.3999 > 1.1     [55mm shaft diameter does not pass yield line]

## Gear Stress Analysis

Choose material for gear:

| Gear Material | Hardness [HB] | E (MPa) | Poisson Ratio | Material Grade |
| --- | --- | --- | --- | --- |
| AISI 1045 Carbon Steel | 400.0000 | 200000.0000 | 0.2850 | 1 |

### Bending

Calculate number of shaft cycles:

t=5 year*52 weekyear*7 dayweek * 8 hrday * 60 minhr=873600 min

Rotation speed n3= 399.7003 RPM.

N=t*n3=349178207.9712 Cycles

Calculate allowable bending stress:

Yn=1.683(N)-0.0323=0.8915	[Stress cycle factor]

Sb=0.533(HB)+88.3=301.5000 MPa	[Material strength for gear grade 1]

kt=kr=1.000	[Temperature not higher than 125 C, 99% reliability]

σb, all=SbYnktkr=268.8011 MPa

Calculate actual bending stress:

Yj=0.420		[Hamrock et al., 2005, N3=27, N4=71]

ka=1.000		[No shock, Electric motor]

ks=1.000		[Module = 5 mm]

km=1.17		[From Hamrock et al, 1999 table] [bw/dp=0.4444, bw=60 mm]

ki=1.000		[Non Idler]

kb=1.000		[Rim thickness for solid disk]

Calculate Kv dynamic factor:

V=πn3d/60=7.4295 [m/s]

Qv=6	[Commercial gear]

B=(12-Qv)2/34=0.8255

A=50+56(1-B)=59.7730

Kv=(A+200VA)B=1.5081

σ b, actual=WtmbwYjkakskmkvkikb=131.9382 MPa

Comparing the allowable to the actual bending stress:

nb=σall/σb=2.0373>1.1

The design is sufficient for bending stress since nb > n = 1.1

### Contact

Calculate allowable contact stress:

HBpinion=500 [HB]

HBgear=400 [HB]

ZN=2.466N-0.056=0.8196		[Cycle factor]

HBpHBg=1.25	[Hardness ratio]

A' = (8.89*10-3)(HBpHBg)-8.29*10-3	[1.2≤HBpHBg=1.25≤1.7]

NgNp=2.6296

CH=1.0+A'(NgNp-1.0)=1.0046	[Hardness Factor, for gear only]

KT=1	[Temperature less than 125 C]

KR=1	[99% reliability]

Sc=2.22*HBg+200=1088 MPa	[Material strength for contact]

σc, all=Sc_ZNCHgKTKR=895.7861 MPa

Calculate actual contact stress:

Ka=1	[No shock, electric motor]

Ks=1	[m = 5 mm]

KE=21-vp2Ep+1-vg2Eg=466563.1271     [Pa]

Km, gear=1.17	[Same from bending]

Kv=1.5081 		[From bending calculation]

ϕ=20 deg; dp=0.135 m; dg=0.355 m

I=πcos(ϕ)sin(ϕ)1+dpdg=0.7315	[Geometry factor]

σc,actual=KEWtbwdpIKaKsKmKv=781.4993 MPa

### Factors of Safety for Bending and Contact

σc,actual=781.4993 MPa

σc, all=895.7861 MPa

σ b, actual=131.9382 MPa

σb, all=268.8011 MPa

nc=σc, allσc,actual=1.1462>1.1		[Safe for contact stress]

nb=σb, allσb,actual=2.0373>1.1		[Safe for bending stress]

## Bearing Selection

h=(5 year)(364 dayyear)(8hrday)=14560 hr	[lifetime hours]

n3 = 399.7003 RPM

L10=60(h)(n3)(10-6)=349.1782 Million Revs

AP=1.000			[No Shock, electric motor]

m=3 [Ball Bearing]

Pr=Max(A,B)=6875.3362 N		[A and B calculated in Force Analysis section]

X=1.000

P=PrX=6875.3362 N 		[No Axial Force]

C=PL101/m=48414.5965 N

Given the calculated C, I specify the following bearing from SKF:

| Bearing Spec | C (N) | C0 (N) | d (m) | D (m) | W (m) |
| --- | --- | --- | --- | --- | --- |
| 6311 | 74100.0000 | 45000.0000 | 0.0550 | 0.1200 | 0.0290 |

Reference: https://www.skf.com/us/products/rolling-bearings/ball-bearings/deep-groove-ball-bearings/productid-6311

This bearing is safe because the bearing specification C is less than our calculated C. Recalculation is not necessary since there is no thrust force and radial factor X is constant, not a function of e or C0.

### Expected Lifetime of Bearing

L=(CP)m=1251.9091 mil rev	[using C=74100 N from SKF]

Life=L(106)(n3)-1(60minhr)-1(8hrday)-1(7dayweek)-1(52weekyr)-1=17.9265 yr

Bearing does not require maintenance for intended operation of 5 years.

## Keyways and Tolerances

Coupling Keyway (50 mm) and Gear Keyway (55 mm):

| Shaft diameter d (mm) | Key width W x height H |
| --- | --- |
| 50 < d <= 58 | 16 x 10 |

Fits:

Bearing and Shaft: Interference fit (H7/p6), bearing should be tightly fit with shaft for fluid rotation.

Gear and Shaft: Transition fit (H7/k6), shaft rotation will be transmitted from gear torque through interlocked key.

Source: https://amesweb.info/fits-tolerances/tolerance-calculator.aspx